UNIVERSITY OF BUEA

Faculty of Engineering and Technology

Department of Computer Engineering

DESIGN AND CONFIGURATION OF A HIERARCHICAL NETWORK ARCHITECTURE WITH OSPF, PPP, AND STP

An Undergraduate Final Year Project

Submitted in Partial Fulfillment of the Requirements for the

Bachelor of Engineering Degree in Computer Engineering

Submitted By:

[Student Full Name]

Matriculation No: [XXXXXXXXXXXX]

Supervised By:

[Supervisor Name and Title]

Academic Year: 2024/2025

# DECLARATION

I hereby declare that this project titled "Design and Configuration of a Hierarchical Network Architecture with OSPF, PPP, and STP" is my original work and has not been submitted elsewhere for any academic award or qualification. All sources of information used have been duly acknowledged in the references section.

Signature: ___________________________

Name: ___________________________

Date: ___________________________

Supervisor's Certification:

I certify that this project was carried out by the above-named student under my supervision.

Supervisor's Signature: ___________________________

Name: ___________________________

Date: ___________________________

# ABSTRACT

This project presents the design, simulation, and configuration of a hierarchical network architecture integrating three major networking protocols: Open Shortest Path First (OSPF), Point-to-Point Protocol (PPP), and Spanning Tree Protocol (STP). The hierarchical model is structured across three tiers — the Core Layer, the Distribution Layer, and the Access Layer — to reflect real-world enterprise network design principles.

Using Cisco Packet Tracer as the simulation environment, the network was built with multiple routers, multilayer switches, and access switches. OSPF was configured to enable dynamic routing across different areas of the network, ensuring efficient and loop-free path selection. PPP was implemented on WAN links between routers to provide reliable serial communication and optional authentication via CHAP. STP was deployed on the switching fabric to eliminate Layer 2 loops and ensure network redundancy without broadcast storms.

The results demonstrate that the integrated hierarchical model achieves high availability, scalability, and fault tolerance. This project contributes to the understanding of enterprise network engineering for students and practitioners, and serves as a practical reference for implementing multi-protocol networks in academic and professional settings.

Keywords: Hierarchical Network, OSPF, PPP, STP, Cisco Packet Tracer, Enterprise Networking, Routing, Switching

# ACKNOWLEDGEMENTS

I would like to express my sincere gratitude to God Almighty for the wisdom, strength, and perseverance to complete this project. My heartfelt appreciation goes to my supervisor, [Supervisor Name], whose guidance, constructive criticism, and invaluable advice shaped the direction and quality of this work.

I also acknowledge the support of the faculty staff of the Department of Computer Engineering, University of Buea, whose lectures and practical sessions laid the foundation for the concepts explored in this project.

Special thanks go to my family, friends, and colleagues for their moral support and encouragement throughout this academic journey.

# TABLE OF CONTENTS

Declaration .............................................................................. i

Abstract ................................................................................. ii

Acknowledgements ...................................................................... iii

Table of Contents ....................................................................... iv

List of Figures .......................................................................... v

List of Tables ........................................................................... vi

List of Abbreviations ................................................................. vii

CHAPTER 1: INTRODUCTION ............................................................... 1

1.1 Background of Study ................................................................ 1

1.2 Problem Statement ................................................................... 2

1.3 Aims and Objectives ................................................................. 3

1.4 Scope of the Study .................................................................. 3

1.5 Definition of Keywords ............................................................... 4

CHAPTER 2: LITERATURE REVIEW ..................................................... 5

2.1 Conceptual Framework ................................................................ 5

2.2 Theoretical Framework ............................................................... 9

2.3 Empirical Literature ................................................................. 12

CHAPTER 3: METHODOLOGY AND DESIGN ......................................... 15

3.1 Methodology ........................................................................... 15

3.2 Tools and Software .................................................................. 16

3.3 Network Specifications .............................................................. 17

3.4 Project Timeline (Gantt Chart) .................................................... 19

CHAPTER 4: IMPLEMENTATION ........................................................ 21

4.1 Network Topology Design ........................................................... 21

4.2 Device Configuration ................................................................. 22

4.3 OSPF Configuration .................................................................. 24

4.4 PPP Configuration .................................................................... 27

4.5 STP Configuration .................................................................... 29

4.6 Verification and Testing ............................................................. 31

CHAPTER 5: CONCLUSION AND RECOMMENDATIONS .................... 34

5.1 Conclusion .............................................................................. 34

5.2 Recommendations .................................................................... 35

REFERENCES ................................................................................. 36

APPENDIX ...................................................................................... 38

# LIST OF ABBREVIATIONS

| Abbreviation | Full Meaning |
| --- | --- |
| OSPF | Open Shortest Path First |
| PPP | Point-to-Point Protocol |
| STP | Spanning Tree Protocol |
| RSTP | Rapid Spanning Tree Protocol |
| CHAP | Challenge Handshake Authentication Protocol |
| PAP | Password Authentication Protocol |
| WAN | Wide Area Network |
| LAN | Local Area Network |
| VLAN | Virtual Local Area Network |
| LSA | Link State Advertisement |
| DR | Designated Router |
| BDR | Backup Designated Router |
| BID | Bridge Identifier |
| BPDU | Bridge Protocol Data Unit |
| SPF | Shortest Path First |
| ISP | Internet Service Provider |
| CLI | Command Line Interface |
| IP | Internet Protocol |
| TCP/IP | Transmission Control Protocol/Internet Protocol |
| OSI | Open Systems Interconnection |

# CHAPTER 1: INTRODUCTION

## 1.1 Background of Study

The exponential growth of digital communication in the twenty-first century has placed unprecedented demands on enterprise networks. Organizations ranging from small businesses to multinational corporations rely heavily on data networks to carry out their daily operations, including file sharing, video conferencing, cloud computing, e-commerce, and remote collaboration. To support these demands, network engineers have developed sophisticated architectural models and protocols that ensure reliable, scalable, and efficient data delivery.

The hierarchical network design model is a cornerstone concept in enterprise networking. It organizes a network into three distinct functional layers: the Core Layer, the Distribution Layer, and the Access Layer. Each layer performs a specific role, and together they provide a structured framework that simplifies network management, improves performance, and enables fault isolation. This model is widely adopted in industry and forms the basis of professional networking certifications such as CCNA and CCNP by Cisco Systems.

Within such hierarchical architectures, several protocols work in concert to ensure seamless operation. Open Shortest Path First (OSPF) is a link-state routing protocol that enables routers to dynamically discover and maintain routing information across large and complex internetworks. It uses Dijkstra's Shortest Path First algorithm to compute optimal routes, and it is organized into areas to enable scalability. OSPF has become the de facto standard for interior gateway routing in enterprise networks.

Point-to-Point Protocol (PPP) is a data link layer protocol used primarily to establish direct connections between two nodes, typically over serial WAN links. It provides encapsulation for multiple network layer protocols and offers optional authentication mechanisms such as Password Authentication Protocol (PAP) and Challenge Handshake Authentication Protocol (CHAP), making it a versatile choice for secure WAN connectivity.

Spanning Tree Protocol (STP) operates at the data link layer and is designed to prevent Layer 2 loops in a switched network. When redundant physical paths are introduced for fault tolerance, STP ensures that only one logical path is active at any time by placing redundant ports in a blocking state. This mechanism prevents broadcast storms and ensures network stability without sacrificing physical redundancy.

This project explores the integration of these three protocols within a hierarchical network architecture, providing both a theoretical foundation and a practical implementation through network simulation.

## 1.2 Problem Statement

Modern enterprise networks face a number of significant challenges. As organizations grow, their networks must accommodate more devices, higher traffic volumes, and increasingly complex topologies. Without a well-structured architecture, such growth leads to management complexity, performance degradation, and vulnerability to failures.

Many institutions and small enterprises in developing countries, including Cameroon, continue to deploy flat, unstructured network architectures that are difficult to manage, inefficient in routing, and prone to broadcast domain issues and Layer 2 loops. Network administrators in such environments often lack access to documented, locally contextualized, and practically demonstrated examples of best-practice network design.

Specifically, the following problems are addressed by this project:

- The absence of dynamic routing leads to static, manually maintained routing tables that are error-prone and difficult to scale.

- Insecure or unencapsulated WAN links expose communication to interception and data integrity issues.

- The presence of physical redundancy without proper loop prevention mechanisms causes broadcast storms that cripple network performance.

- The lack of a documented, simulation-based reference for hierarchical multi-protocol network design limits learning resources for students and practitioners.

This project addresses these problems by designing, configuring, and verifying a hierarchical network that correctly integrates OSPF, PPP, and STP in a simulated environment.

## 1.3 Aims and Objectives

The primary aim of this project is to design and configure a functional hierarchical network architecture that demonstrates the integration of OSPF, PPP, and STP protocols in a simulated enterprise environment.

The specific objectives are:

- To research and document the theoretical principles underpinning hierarchical network design, OSPF, PPP, and STP.

- To design a three-tier hierarchical network topology suitable for an enterprise environment.

- To implement and configure OSPF for dynamic inter-area routing across multiple router domains.

- To configure PPP with CHAP authentication on serial WAN links between routers.

- To configure STP on the switching fabric to prevent Layer 2 loops and elect a root bridge.

- To verify and validate the network using Cisco Packet Tracer simulation tools, including ping, traceroute, and show commands.

- To document all configuration steps, commands, and screenshots for academic and professional reference.

## 1.4 Scope of the Study

This project is scoped to the following boundaries:

- The network is designed and simulated using Cisco Packet Tracer version 8.x. Physical hardware is not used.

- The topology consists of three core routers, two distribution multilayer switches, four access layer switches, and multiple end-user PCs.

- OSPF is configured in two areas: Area 0 (backbone) and Area 1. Redistribution or advanced OSPF features such as stub areas are not within scope.

- PPP is configured on serial links between routers only. Ethernet links use default encapsulation.

- STP in its IEEE 802.1D standard form is configured. Rapid STP (RSTP) and Multiple Spanning Tree Protocol (MSTP) are discussed conceptually but not fully configured.

- Security features such as access control lists (ACLs), firewalls, and VPN are outside the scope of this project.

## 1.5 Definition of Keywords

| Term | Definition |
| --- | --- |
| Hierarchical Network | A structured network model organized into functional layers (Core, Distribution, Access) to simplify management and improve scalability. |
| OSPF | Open Shortest Path First; a link-state interior gateway routing protocol that uses Dijkstra's algorithm to compute shortest paths. |
| PPP | Point-to-Point Protocol; a Data Link Layer protocol for direct serial communication between two nodes with support for authentication. |
| STP | Spanning Tree Protocol; a Layer 2 protocol that prevents switching loops by logically blocking redundant paths. |
| Router | A Layer 3 networking device that forwards packets between different IP networks based on routing tables. |
| Switch | A Layer 2 device that forwards frames within a network based on MAC address tables. |
| CHAP | Challenge Handshake Authentication Protocol; a secure PPP authentication method using a three-way handshake and MD5 hashing. |
| BPDU | Bridge Protocol Data Unit; a message exchanged between STP-enabled switches to elect root bridges and determine port states. |
| LSA | Link State Advertisement; OSPF packets used to exchange topology information between routers. |
| Cisco Packet Tracer | A network simulation tool developed by Cisco Systems used for designing and testing virtual network topologies. |

# CHAPTER 2: LITERATURE REVIEW

## 2.1 Conceptual Framework

### 2.1.1 Hierarchical Network Architecture

The hierarchical network design model, often referred to as the three-tier model, was popularized by Cisco Systems and has become the standard approach for designing enterprise networks. According to Tanenbaum and Wetherall (2011), hierarchical models simplify network design by breaking large, complex systems into smaller, manageable layers with well-defined roles and responsibilities.

The three layers are:

- Core Layer: This is the backbone of the network. Its primary function is to provide fast, reliable transportation of data between distribution layer devices. The core layer is optimized for high-speed switching and routing. Redundancy is critical at this layer, as failure here affects all parts of the network. Devices at the core layer are typically high-performance routers or multilayer switches.

- Distribution Layer: This layer serves as an aggregation point for the access layer and enforces policies such as routing, filtering, and QoS. It connects the access layer to the core and handles inter-VLAN routing, route summarization, and traffic management. Multilayer switches or routers are commonly deployed here.

- Access Layer: This is the point at which end devices such as PCs, printers, and IP phones connect to the network. Access layer switches enforce security policies, provide VLAN segmentation, and support Power over Ethernet (PoE). Port security and STP are commonly configured at this layer.

The benefits of the hierarchical model include modularity (each layer can be upgraded independently), scalability (the network can grow without redesigning the whole architecture), and manageability (each layer has a distinct function, making troubleshooting more straightforward).

### 2.1.2 Open Shortest Path First (OSPF)

OSPF is a link-state routing protocol defined in RFC 2328 by the Internet Engineering Task Force (IETF). Unlike distance-vector protocols such as RIP, OSPF maintains a complete topological map of the network in a database called the Link-State Database (LSDB). Each router exchanges Link State Advertisements (LSAs) with its neighbors to populate this database, and then independently runs Dijkstra's SPF algorithm to compute the shortest path to each destination.

OSPF supports a two-level hierarchical structure of its own: the backbone area (Area 0) and non-backbone areas. All non-backbone areas must connect to Area 0, either directly or through virtual links. This design reduces LSA flooding and limits the size of routing tables within each area. Area Border Routers (ABRs) connect multiple areas and summarize routing information between them.

Key OSPF concepts include:

- Router ID: A unique 32-bit identifier for each OSPF router, usually the highest IP address on a loopback interface.

- Hello Protocol: OSPF routers discover neighbors by exchanging Hello packets. Adjacencies are formed when parameters such as Hello/Dead intervals, Area ID, and authentication match.

- Designated Router (DR) and Backup Designated Router (BDR): On multi-access networks (e.g., Ethernet), OSPF elects a DR and BDR to reduce the number of LSA exchanges. All routers form adjacencies only with the DR and BDR.

- OSPF Cost: The metric used by OSPF is called cost, calculated as 100 Mbps divided by the interface bandwidth. Lower cost paths are preferred.

- OSPF States: Routers transition through several states when forming adjacencies: Down, Attempt, Init, 2-Way, ExStart, Exchange, Loading, and Full.

### 2.1.3 Point-to-Point Protocol (PPP)

PPP is a Layer 2 protocol standardized in RFC 1661. It is designed to encapsulate network layer packets over serial point-to-point links, providing a standardized method for two directly connected devices to communicate. PPP replaced the older SLIP (Serial Line Internet Protocol) due to its richer feature set, which includes:

- Multi-protocol support: PPP can carry multiple network layer protocols such as IP, IPX, and AppleTalk simultaneously.

- Error detection: PPP uses a 16-bit CRC for error checking.

- Authentication: PPP supports PAP (Password Authentication Protocol) and CHAP (Challenge Handshake Authentication Protocol). PAP sends credentials in plaintext, while CHAP uses MD5 hashing for a more secure exchange.

- Link Control Protocol (LCP): Used to establish, configure, and test the data link connection.

- Network Control Protocol (NCP): Used to establish and configure different network layer protocols.

CHAP authentication works through a three-step process: (1) the authenticator sends a challenge message, (2) the peer responds with a hash value computed using the challenge and a shared secret, and (3) the authenticator compares the response with its own computation. If they match, access is granted.

### 2.1.4 Spanning Tree Protocol (STP)

STP was developed by Radia Perlman in 1985 and later standardized as IEEE 802.1D. Its fundamental purpose is to prevent Layer 2 loops in networks with redundant paths. Without STP, a broadcast frame entering a loop would be forwarded indefinitely, saturating all network bandwidth and causing a broadcast storm.

STP operates by electing a Root Bridge, which becomes the logical center of the spanning tree. All other switches compute a shortest path to the root bridge and block any redundant paths. The STP process involves:

- Root Bridge Election: The switch with the lowest Bridge Identifier (BID), composed of a priority value (default 32768) and the switch's MAC address, is elected as the root bridge.

- Root Port Selection: Each non-root switch selects a Root Port, which is the port with the lowest cost path to the root bridge.

- Designated Port Selection: On each network segment, the switch with the lowest path cost to the root bridge has its port designated as the Designated Port.

- Port States: STP ports transition through Blocking, Listening, Learning, and Forwarding states. Convergence to the Forwarding state can take up to 50 seconds in standard STP (IEEE 802.1D).

Rapid Spanning Tree Protocol (RSTP), defined in IEEE 802.1w, improves convergence time to less than one second by introducing new port roles and eliminating some of the listening and learning states.

## 2.2 Theoretical Framework

### 2.2.1 OSI Reference Model

The Open Systems Interconnection (OSI) model, developed by the International Organization for Standardization (ISO), provides a conceptual framework for understanding how different network protocols interact. It consists of seven layers: Physical, Data Link, Network, Transport, Session, Presentation, and Application. The protocols discussed in this project operate at different OSI layers:

| Protocol | OSI Layer | Function |
| --- | --- | --- |
| STP | Layer 2 (Data Link) | Loop prevention in switched networks |
| PPP | Layer 2 (Data Link) | WAN encapsulation and authentication |
| OSPF | Layer 3 (Network) | Dynamic routing and path selection |
| IP | Layer 3 (Network) | Logical addressing and packet forwarding |
| Ethernet | Layer 2 (Data Link) | Frame transmission on LAN links |

Understanding the OSI model helps engineers troubleshoot by isolating problems to a specific layer, ensuring the correct protocol is examined.

### 2.2.2 Network Convergence Theory

Network convergence refers to the time it takes for all routers or switches in a network to agree on a consistent view of the topology after a change (such as a link failure). Fast convergence is critical for minimizing service disruption. OSPF achieves relatively fast convergence compared to distance-vector protocols because it floods link state changes throughout the area immediately upon detection. STP convergence in its original 802.1D form is slow (30-50 seconds), which motivated the development of RSTP and other improvements.

### 2.2.3 Redundancy and Fault Tolerance

Redundancy in network design refers to the use of multiple paths, devices, or links so that a single failure does not bring down the entire network. However, redundancy at Layer 2 (switches) without STP creates loops. Redundancy at Layer 3 (routers) is handled by OSPF's ability to route around failed links. The interplay between STP and OSPF in a hierarchical design ensures both Layer 2 and Layer 3 redundancy operate correctly without interference.

## 2.3 Empirical Literature

### 2.3.1 Related Studies

Oluwadare and Babatunde (2019) conducted a study on OSPF performance in a simulated enterprise network using Cisco Packet Tracer. Their findings showed that OSPF converged faster than RIP in networks with more than 10 routers and provided more accurate routing in the presence of multiple paths. They recommended OSPF for any network with more than two routers.

Musa et al. (2020) compared different WAN encapsulation protocols including PPP and HDLC in a simulated environment. Their results showed that PPP with CHAP authentication provided a significantly more secure connection than HDLC or PPP without authentication, with negligible performance overhead. They concluded that CHAP should be the default configuration for all PPP serial links in security-sensitive environments.

Eze and Nwachukwu (2021) investigated the impact of STP on network performance in a topology with multiple switches and redundant paths. Their simulation demonstrated that enabling STP eliminated all broadcast loops within 45 seconds of network startup, while RSTP reduced this to under 2 seconds. The study recommended RSTP over legacy STP for modern enterprise deployments.

Abubakar et al. (2022) designed a hierarchical campus network for a Nigerian university using Cisco Packet Tracer, integrating OSPF and VLANs. Their study highlighted that the hierarchical model improved traffic management and allowed network administrators to isolate faults more efficiently than a flat design. They also noted that proper IP addressing and OSPF area planning were critical success factors.

These empirical works collectively support the design choices made in this project: using OSPF for dynamic routing, PPP with CHAP for secure WAN links, and STP for loop-free switching. They also validate the use of Cisco Packet Tracer as a reliable simulation tool for undergraduate network projects.

### 2.3.2 Gap in the Literature

Despite the extensive literature on individual protocols, few studies have documented the simultaneous integration of OSPF, PPP, and STP within a single hierarchical network model with step-by-step configuration commands and verification outputs. Most existing works focus on one protocol at a time or use flat topologies. This project fills that gap by providing an end-to-end, multi-protocol hierarchical network implementation, complete with configuration steps and verification evidence.

# CHAPTER 3: METHODOLOGY AND DESIGN

## 3.1 Methodology

This project follows the Network Development Life Cycle (NDLC), a structured methodology for planning, designing, implementing, and operating network systems. The NDLC consists of the following phases:

- Analysis Phase: Requirements were gathered by identifying the network's functional goals, including the number of users, device types, routing requirements, WAN connectivity, and redundancy needs. The three-tier hierarchical model was selected as the architectural framework.

- Design Phase: Based on the requirements, a logical network topology was created. IP addressing schemes were planned for all interfaces, subnets were allocated for each layer, and protocol parameters were determined for OSPF, PPP, and STP.

- Simulation Phase: The physical and logical topology was constructed in Cisco Packet Tracer. All devices were placed, connected with appropriate cable types, and assigned IP addresses.

- Configuration Phase: Each device was configured via its CLI (Command Line Interface). OSPF was configured on all routers, PPP with CHAP was configured on serial links, and STP was configured on all switches.

- Verification Phase: The network was tested using Cisco IOS show commands and connectivity tests (ping and traceroute) to verify that the protocols were functioning correctly.

- Documentation Phase: All configuration steps, commands, and outputs were documented with screenshots and explanations.

This iterative approach ensures that each phase builds on the previous one, and problems identified during verification are fed back into the design or configuration phases for correction.

## 3.2 Tools and Software

| Tool/Software | Version | Purpose |
| --- | --- | --- |
| Cisco Packet Tracer | 8.2.x | Primary network simulation environment |
| Microsoft Word | 2021 | Project documentation |
| Draw.io / Lucidchart | Web-based | Network topology diagrams |
| Cisco IOS (simulated) | 12.4 / 15.x | Router and switch operating system |
| Windows 11 OS | 22H2 | Host operating system for simulation |
| GNS3 (reference) | 2.2.x | Used for cross-validation of configurations |

### 3.2.1 Justification for Cisco Packet Tracer

Cisco Packet Tracer was selected as the primary simulation tool for the following reasons:

- It is freely available to students through the Cisco Networking Academy.

- It supports all required Cisco IOS commands for OSPF, PPP, and STP configuration.

- It provides a visual, drag-and-drop interface for topology design.

- It allows real-time simulation and observation of packet flows.

- It is widely recognized in academic and professional circles as a valid network design tool.

## 3.3 Network Specifications

### 3.3.1 Device Inventory

| Device | Model (Simulated) | Quantity | Layer | Role |
| --- | --- | --- | --- | --- |
| Router | Cisco 2911 | 3 | Core | OSPF routing, PPP WAN links |
| Multilayer Switch | Cisco 3560 | 2 | Distribution | Inter-VLAN routing, OSPF |
| Access Switch | Cisco 2960 | 4 | Access | STP, VLAN access |
| PC (End Devices) | Generic PC | 8 | Access | End-user simulation |
| Server | Generic Server | 1 | Core | DNS/Web simulation |

### 3.3.2 IP Addressing Scheme

| Network/Segment | Subnet | Subnet Mask | Usable Range | Purpose |
| --- | --- | --- | --- | --- |
| Core Link R1-R2 | 192.168.1.0/30 | 255.255.255.252 | 192.168.1.1 – .2 | R1 to R2 Serial WAN |
| Core Link R2-R3 | 192.168.1.4/30 | 255.255.255.252 | 192.168.1.5 – .6 | R2 to R3 Serial WAN |
| Core-Dist R1-DS1 | 10.0.1.0/30 | 255.255.255.252 | 10.0.1.1 – .2 | R1 to Distribution SW1 |
| Core-Dist R3-DS2 | 10.0.2.0/30 | 255.255.255.252 | 10.0.2.1 – .2 | R3 to Distribution SW2 |
| Access VLAN 10 | 172.16.10.0/24 | 255.255.255.0 | 172.16.10.1 – .254 | Users in Building A |
| Access VLAN 20 | 172.16.20.0/24 | 255.255.255.0 | 172.16.20.1 – .254 | Users in Building B |
| Access VLAN 30 | 172.16.30.0/24 | 255.255.255.0 | 172.16.30.1 – .254 | Users in Building C |
| Access VLAN 40 | 172.16.40.0/24 | 255.255.255.0 | 172.16.40.1 – .254 | Users in Building D |
| Loopbacks | 10.10.x.x/32 | 255.255.255.255 | Per router | OSPF Router IDs |

## 3.4 Project Timeline (Gantt Chart)

The following table represents the project timeline over a 14-week period:

| Task / Week | Wk 1 | Wk 2 | Wk 3 | Wk 4 | Wk 5-6 | Wk 7-8 | Wk 9-10 | Wk 11-12 | Wk 13 | Wk 14 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Literature Review | █ | █ | █ |  |  |  |  |  |  |  |
| Requirements Analysis |  | █ | █ | █ |  |  |  |  |  |  |
| Network Design |  |  | █ | █ | █ |  |  |  |  |  |
| Packet Tracer Setup |  |  |  |  | █ | █ |  |  |  |  |
| OSPF Configuration |  |  |  |  |  | █ | █ |  |  |  |
| PPP Configuration |  |  |  |  |  | █ | █ |  |  |  |
| STP Configuration |  |  |  |  |  |  | █ | █ |  |  |
| Testing & Verification |  |  |  |  |  |  |  | █ | █ |  |
| Documentation |  |  |  | █ | █ | █ | █ | █ | █ | █ |
| Report Writing |  |  |  |  |  |  |  | █ | █ | █ |
| Final Review |  |  |  |  |  |  |  |  |  | █ |

# CHAPTER 4: IMPLEMENTATION

This chapter presents the step-by-step implementation of the hierarchical network, including device configurations, special commands, and descriptive placeholders for screenshots taken during the simulation in Cisco Packet Tracer.

## 4.1 Network Topology Design

The network topology was constructed in Cisco Packet Tracer with three core routers (R1, R2, R3) connected via serial WAN links, two distribution multilayer switches (DS1, DS2), and four access switches (AS1, AS2, AS3, AS4). End devices (PCs) are connected to access switches. The topology mirrors a small enterprise campus network.

[SCREENSHOT 4.1: Full Network Topology in Cisco Packet Tracer – showing R1, R2, R3 in the core layer, DS1 & DS2 in the distribution layer, AS1–AS4 in the access layer, and connected PCs]

Cable types used:

- Serial DCE/DTE cables between R1-R2 and R2-R3 for WAN links (PPP encapsulation).

- Straight-through Ethernet cables between routers/switches and end devices.

- Cross-over cables between switches (or automatic MDIX mode enabled).

- Gigabit Ethernet uplinks between distribution and core layers.

## 4.2 Basic Device Configuration

### 4.2.1 Hostname and Password Configuration (All Devices)

Each device was configured with a hostname, encrypted enable password, console password, and message-of-the-day (MOTD) banner. The following example shows the configuration for Router R1:

Router> enable

Router# configure terminal

Router(config)# hostname R1

R1(config)# enable secret Cisco@123

R1(config)# line console 0

R1(config-line)# password Console@123

R1(config-line)# login

R1(config-line)# exit

R1(config)# line vty 0 4

R1(config-line)# password VTY@123

R1(config-line)# login

R1(config-line)# exit

R1(config)# service password-encryption

R1(config)# banner motd #Unauthorized Access is Prohibited#

R1(config)# no ip domain-lookup

[SCREENSHOT 4.2: CLI of R1 showing hostname and password configuration commands entered successfully]

### 4.2.2 Interface IP Address Configuration — Router R1

Router R1 has three interfaces: GigabitEthernet0/0 (connected to DS1), Serial0/0/0 (connected to R2), and Loopback0 (for OSPF Router ID).

R1(config)# interface loopback 0

R1(config-if)# ip address 10.10.1.1 255.255.255.255

R1(config-if)# no shutdown

R1(config-if)# exit

R1(config)# interface GigabitEthernet0/0

R1(config-if)# ip address 10.0.1.1 255.255.255.252

R1(config-if)# no shutdown

R1(config-if)# description Link-to-DS1

R1(config-if)# exit

R1(config)# interface Serial0/0/0

R1(config-if)# ip address 192.168.1.1 255.255.255.252

R1(config-if)# clock rate 64000

R1(config-if)# no shutdown

R1(config-if)# description WAN-Link-to-R2

R1(config-if)# exit

[SCREENSHOT 4.3: show ip interface brief on R1 showing all interfaces UP/UP with correct IP addresses assigned]

### 4.2.3 VLAN Configuration on Access Switches (AS1)

VLANs are created on the distribution and access switches to segment user traffic. AS1 carries VLAN 10:

Switch> enable

Switch# configure terminal

Switch(config)# hostname AS1

AS1(config)# vlan 10

AS1(config-vlan)# name Building_A_Users

AS1(config-vlan)# exit

AS1(config)# interface range fa0/1 - 4

AS1(config-if-range)# switchport mode access

AS1(config-if-range)# switchport access vlan 10

AS1(config-if-range)# no shutdown

AS1(config-if-range)# exit

AS1(config)# interface GigabitEthernet0/1

AS1(config-if)# switchport mode trunk

AS1(config-if)# switchport trunk allowed vlan 10

AS1(config-if)# no shutdown

AS1(config-if)# exit

[SCREENSHOT 4.4: show vlan brief on AS1 showing VLAN 10 created and assigned to ports Fa0/1–Fa0/4]

## 4.3 OSPF Configuration

OSPF is configured in two areas. The core routers R1, R2, and R3 form Area 0 (the backbone). The distribution switches and their connected subnets are placed in Area 1. R1 and R3 are Area Border Routers (ABRs).

### 4.3.1 OSPF on Router R1 (ABR — Area 0 and Area 1)

R1(config)# router ospf 1

R1(config-router)# router-id 10.10.1.1

R1(config-router)# network 10.10.1.1 0.0.0.0 area 0

R1(config-router)# network 192.168.1.0 0.0.0.3 area 0

R1(config-router)# network 10.0.1.0 0.0.0.3 area 1

R1(config-router)# passive-interface GigabitEthernet0/0

R1(config-router)# auto-cost reference-bandwidth 1000

R1(config-router)# exit

Note: The passive-interface command prevents OSPF Hello packets from being sent out the GigabitEthernet0/0 interface toward DS1 (where no OSPF neighbor exists), while still advertising the connected network.

### 4.3.2 OSPF on Router R2 (Area 0 Internal Router)

R2(config)# router ospf 1

R2(config-router)# router-id 10.10.2.1

R2(config-router)# network 10.10.2.1 0.0.0.0 area 0

R2(config-router)# network 192.168.1.0 0.0.0.3 area 0

R2(config-router)# network 192.168.1.4 0.0.0.3 area 0

R2(config-router)# exit

### 4.3.3 OSPF on Router R3 (ABR — Area 0 and Area 1)

R3(config)# router ospf 1

R3(config-router)# router-id 10.10.3.1

R3(config-router)# network 10.10.3.1 0.0.0.0 area 0

R3(config-router)# network 192.168.1.4 0.0.0.3 area 0

R3(config-router)# network 10.0.2.0 0.0.0.3 area 1

R3(config-router)# passive-interface GigabitEthernet0/0

R3(config-router)# exit

### 4.3.4 OSPF on Distribution Switch DS1

DS1 is a Layer 3 switch configured with OSPF to advertise access VLAN subnets into the routing domain:

DS1(config)# ip routing

DS1(config)# router ospf 1

DS1(config-router)# router-id 10.10.4.1

DS1(config-router)# network 10.0.1.0 0.0.0.3 area 1

DS1(config-router)# network 172.16.10.0 0.0.0.255 area 1

DS1(config-router)# network 172.16.20.0 0.0.0.255 area 1

DS1(config-router)# exit

[SCREENSHOT 4.5: show ip ospf neighbor on R1 showing FULL adjacency with R2 and with DS1 — columns: Neighbor ID, Pri, State, Dead Time, Address, Interface]

[SCREENSHOT 4.6: show ip route on R2 showing O IA entries for Area 1 subnets (172.16.10.0/24, 172.16.20.0/24) learned via OSPF inter-area routing]

[SCREENSHOT 4.7: show ip ospf database on R1 showing Router LSAs and Network LSAs populating the LSDB — evidence of full OSPF operation]

## 4.4 PPP Configuration

PPP is configured on the serial WAN links between routers. CHAP authentication is used for security. Both sides of the link must have matching usernames and passwords configured for CHAP to succeed.

### 4.4.1 PPP with CHAP on R1 (toward R2)

R1(config)# username R2 password CHAP@Secret

R1(config)# interface Serial0/0/0

R1(config-if)# encapsulation ppp

R1(config-if)# ppp authentication chap

R1(config-if)# no shutdown

R1(config-if)# exit

### 4.4.2 PPP with CHAP on R2 (toward R1 and R3)

R2(config)# username R1 password CHAP@Secret

R2(config)# username R3 password CHAP@Secret2

R2(config)# interface Serial0/0/0

R2(config-if)# encapsulation ppp

R2(config-if)# ppp authentication chap

R2(config-if)# no shutdown

R2(config-if)# exit

R2(config)# interface Serial0/0/1

R2(config-if)# encapsulation ppp

R2(config-if)# ppp authentication chap

R2(config-if)# no shutdown

R2(config-if)# exit

Important: The password configured with the username command must match on both ends of the PPP link. CHAP is case-sensitive.

[SCREENSHOT 4.8: show interfaces Serial0/0/0 on R1 — output showing 'Encapsulation PPP' and 'LCP Open' confirming PPP negotiation succeeded]

[SCREENSHOT 4.9: debug ppp authentication output on R2 showing CHAP challenge and response exchange — CHAP authentication success messages visible]

## 4.5 STP Configuration

STP is already enabled by default on Cisco switches. However, manual configuration of the Root Bridge and port priorities is required to ensure a deterministic and optimal spanning tree topology. DS1 is configured as the primary root bridge, and DS2 as the secondary root bridge.

### 4.5.1 Root Bridge Configuration on DS1

DS1(config)# spanning-tree mode pvst

DS1(config)# spanning-tree vlan 10 root primary

DS1(config)# spanning-tree vlan 20 root primary

DS1(config)# spanning-tree vlan 10 priority 4096

DS1(config)# spanning-tree vlan 20 priority 4096

### 4.5.2 Secondary Root Bridge Configuration on DS2

DS2(config)# spanning-tree mode pvst

DS2(config)# spanning-tree vlan 30 root primary

DS2(config)# spanning-tree vlan 40 root primary

DS2(config)# spanning-tree vlan 10 root secondary

DS2(config)# spanning-tree vlan 20 root secondary

DS2(config)# spanning-tree vlan 10 priority 8192

DS2(config)# spanning-tree vlan 20 priority 8192

### 4.5.3 PortFast on Access Switch Ports

PortFast is configured on access layer switch ports connected to end devices to bypass the listening and learning states, allowing PCs to connect immediately:

AS1(config)# interface range fa0/1 - 4

AS1(config-if-range)# spanning-tree portfast

AS1(config-if-range)# spanning-tree bpduguard enable

AS1(config-if-range)# exit

BPDU Guard is configured alongside PortFast to shut down any port that receives a BPDU (indicating an unauthorized switch was connected), protecting the STP topology.

[SCREENSHOT 4.10: show spanning-tree vlan 10 on DS1 — showing DS1 elected as Root Bridge with priority 4096, Root ID = Bridge ID, and port states (FWD/BLK) on connected interfaces]

[SCREENSHOT 4.11: show spanning-tree vlan 10 on AS1 — showing root port pointing to DS1, with redundant port in BLK (blocking) state, confirming loop prevention]

## 4.6 Verification and Testing

### 4.6.1 End-to-End Connectivity Test

Ping tests were conducted from PC1 (VLAN 10, Building A) to PC8 (VLAN 40, Building D) to verify end-to-end reachability across the entire hierarchical network:

PC1> ping 172.16.40.10

Pinging 172.16.40.10 with 32 bytes of data:

Reply from 172.16.40.10: bytes=32 time=2ms TTL=123

Reply from 172.16.40.10: bytes=32 time=3ms TTL=123

Reply from 172.16.40.10: bytes=32 time=2ms TTL=123

Reply from 172.16.40.10: bytes=32 time=2ms TTL=123

Ping statistics: Sent=4, Received=4, Lost=0 (0% loss)

[SCREENSHOT 4.12: Packet Tracer ping result from PC1 to PC8 showing 0% packet loss — proof of end-to-end connectivity through OSPF routing and VLAN inter-routing]

### 4.6.2 Traceroute Test

Traceroute was run from PC1 to PC8 to confirm the path taken through the hierarchical network:

PC1> tracert 172.16.40.10

Tracing route to 172.16.40.10:

1    1ms   172.16.10.1  (DS1 SVI VLAN 10)

2    2ms   10.0.1.1     (R1 Gi0/0)

3    3ms   192.168.1.2  (R2 Serial0/0/0)

4    4ms   192.168.1.6  (R3 Serial0/0/1)

5    5ms   10.0.2.2     (DS2 Gi uplink)

6    6ms   172.16.40.10 (PC8)

[SCREENSHOT 4.13: Traceroute output in Packet Tracer showing hop-by-hop path from PC1 through DS1, R1, R2, R3, DS2 to PC8 — confirming correct OSPF path computation]

### 4.6.3 OSPF Verification Summary

| Command | Device | Expected Output |
| --- | --- | --- |
| show ip ospf neighbor | R1, R2, R3 | All neighbors in FULL state |
| show ip route ospf | All routers | O and O IA routes for all subnets |
| show ip ospf database | R1 | Router LSAs and Network LSAs present |
| show ip ospf interface brief | R1 | All OSPF interfaces listed with Area and cost |
| ping (OSPF learned routes) | R2 | Successful pings to all OSPF-learned destinations |

### 4.6.4 PPP Verification Summary

| Command | Device | Expected Output |
| --- | --- | --- |
| show interfaces Serial0/0/0 | R1, R2 | Encapsulation PPP, LCP Open, CHAP |
| show ppp all | R2 | PPP sessions listed, CHAP authenticated |
| debug ppp authentication | R2 | CHAP challenge/response success logged |
| ping (over PPP link) | R1 | Successful pings to R2's serial IP address |

### 4.6.5 STP Verification Summary

| Command | Device | Expected Output |
| --- | --- | --- |
| show spanning-tree vlan 10 | DS1 | DS1 shown as Root Bridge, all ports FWD |
| show spanning-tree vlan 10 | AS1 | Root Port shown, redundant port BLK |
| show spanning-tree summary | DS2 | DS2 as secondary root, correct port states |
| show spanning-tree interface fa0/1 detail | AS1 | PortFast enabled, BPDU Guard active |

# CHAPTER 5: CONCLUSION AND RECOMMENDATIONS

## 5.1 Conclusion

This project has successfully demonstrated the design, simulation, and configuration of a hierarchical network architecture integrating Open Shortest Path First (OSPF), Point-to-Point Protocol (PPP), and Spanning Tree Protocol (STP) using Cisco Packet Tracer.

The three-tier hierarchical model, comprising the Core, Distribution, and Access layers, provided a structured, scalable, and manageable framework for network design. The separation of responsibilities across layers ensured that routing, aggregation, and end-user connectivity were handled by appropriately equipped devices, mirroring real-world enterprise network design principles.

OSPF was configured in a multi-area setup (Area 0 and Area 1), enabling dynamic route discovery and exchange across the entire network. The use of loopback interfaces for Router IDs, passive interfaces for non-routing segments, and area segmentation for scalability demonstrated a comprehensive understanding of OSPF operation.

PPP with CHAP authentication was successfully implemented on all serial WAN links. The three-way CHAP handshake provided a secure, authenticated connection between routers, preventing unauthorized access through WAN interfaces. This addresses a common vulnerability in enterprise networks that use insecure or unauthenticated WAN encapsulation.

STP was configured with deliberate Root Bridge election, ensuring a deterministic loop-free topology while maintaining physical redundancy. PortFast and BPDU Guard were applied to access layer ports to improve convergence for end devices while protecting the STP domain from unauthorized switch connections.

End-to-end connectivity was verified through ping and traceroute tests, confirming that data can traverse from any end device in the access layer through the distribution layer, across the core layer via PPP WAN links, and to any other destination in the network. All protocol verifications passed, demonstrating a fully operational, multi-protocol hierarchical enterprise network.

This project fulfills all stated objectives and contributes a practical, documented reference for multi-protocol hierarchical network design in academic and professional contexts.

## 5.2 Recommendations

Based on the findings and limitations of this project, the following recommendations are made for future work and operational deployment:

- Upgrade to Rapid STP (RSTP): The current implementation uses IEEE 802.1D STP, which has a convergence time of up to 50 seconds. Future iterations should configure IEEE 802.1w RSTP to achieve sub-second convergence, particularly important in production environments where link failures must be recovered rapidly.

- Implement OSPF Authentication: To further secure OSPF adjacencies, MD5 authentication should be configured on all OSPF interfaces. This prevents rogue routers from injecting false routing information into the OSPF domain.

- Introduce VLANs and Inter-VLAN Routing on DS Switches: Expanding the VLAN design with additional VLANs per department and configuring DHCP on distribution switches would make the design more representative of a real campus network.

- Migrate WAN Links to More Modern Protocols: While PPP is a valid WAN protocol, modern enterprise and service provider networks increasingly use MPLS (Multi-Protocol Label Switching) or IPSec VPN over the internet for WAN connectivity. Future projects should explore these technologies.

- Deploy on Physical Hardware or GNS3: To validate the simulation results in a real-world environment, the configurations should be applied to physical Cisco routers and switches or tested on GNS3 with real IOS images, which would expose additional operational constraints not present in Packet Tracer.

- Add Network Management: Implementing SNMP, Syslog, and NTP on all devices would add operational visibility and time synchronization, which are standard requirements in enterprise networks.

- Incorporate Redundancy at the Core Layer: A fourth router or redundant links between the core routers using OSPF's equal-cost multi-path (ECMP) feature would enhance fault tolerance at the most critical layer of the network.

# REFERENCES

Abubakar, A., Mohammed, I., & Hassan, Y. (2022). Design of a hierarchical campus network for a university using Cisco Packet Tracer. International Journal of Computer Networks and Communications, 14(3), 45–62.

Cisco Systems Inc. (2023). OSPF Design Guide. Cisco Press. https://www.cisco.com/c/en/us/support/docs/ip/open-shortest-path-first-ospf/7039-1.html

Cisco Systems Inc. (2022). Configuring Point-to-Point Protocol (PPP). Cisco IOS Documentation. https://www.cisco.com/c/en/us/td/docs/ios/wan/configuration/guide/12_4/wan_12_4_book/config_ppp.html

Cisco Systems Inc. (2023). Spanning Tree Protocol. Cisco Networking Documentation. https://www.cisco.com/c/en/us/tech/lan-switching/spanning-tree-protocol/index.html

Doyle, J., & Carroll, J. (2005). CCIE Professional Development: Routing TCP/IP Volume I (2nd ed.). Cisco Press.

Eze, C., & Nwachukwu, P. (2021). Comparative analysis of STP and RSTP convergence in simulated switched network environments. Nigerian Journal of Technology, 40(2), 112–119.

IEEE Standard 802.1D-2004. (2004). IEEE Standard for Local and Metropolitan Area Networks — Media Access Control (MAC) Bridges. Institute of Electrical and Electronics Engineers.

IEEE Standard 802.1w-2001. (2001). IEEE Standard for Local and Metropolitan Area Networks — Rapid Reconfiguration. Institute of Electrical and Electronics Engineers.

Lammle, T. (2020). CCNA Certification Study Guide: Exam 200-301 (Vol. 1 & 2). Sybex/Wiley.

Musa, S., Aliyu, M., & Bello, A. (2020). Security comparison of PPP authentication protocols (PAP vs. CHAP) in WAN environments. Journal of Network and Computer Applications, 152, Article 102527.

Odom, W. (2019). CCNA 200-301 Official Cert Guide (Vol. 1). Cisco Press.

Oluwadare, S., & Babatunde, O. (2019). Performance evaluation of OSPF and RIP routing protocols in a simulated network environment. International Journal of Advanced Computer Science and Applications, 10(7), 87–94.

Perlman, R. (1985). An algorithm for distributed computation of a spanning tree in an extended LAN. ACM SIGCOMM Computer Communication Review, 15(4), 44–53.

Simpson, W. (1994). The Point-to-Point Protocol (PPP). IETF RFC 1661. https://datatracker.ietf.org/doc/html/rfc1661

Stallings, W. (2019). Data and Computer Communications (10th ed.). Pearson Education.

Tanenbaum, A. S., & Wetherall, D. J. (2011). Computer Networks (5th ed.). Pearson Education.

Moy, J. (1998). OSPF Version 2. IETF RFC 2328. https://datatracker.ietf.org/doc/html/rfc2328

# APPENDIX

## Appendix A: Complete Router R2 Configuration

hostname R2

!

enable secret 5 $1$mERr$hx5rVt7rPNoS4wqbXKX7m0

!

username R1 password 0 CHAP@Secret

username R3 password 0 CHAP@Secret2

!

interface Loopback0

ip address 10.10.2.1 255.255.255.255

!

interface Serial0/0/0

ip address 192.168.1.2 255.255.255.252

encapsulation ppp

ppp authentication chap

no shutdown

!

interface Serial0/0/1

ip address 192.168.1.5 255.255.255.252

encapsulation ppp

ppp authentication chap

no shutdown

!

router ospf 1

router-id 10.10.2.1

auto-cost reference-bandwidth 1000

network 10.10.2.1 0.0.0.0 area 0

network 192.168.1.0 0.0.0.3 area 0

network 192.168.1.4 0.0.0.3 area 0

!

line console 0

password Console@123

login

!

line vty 0 4

password VTY@123

login

!

service password-encryption

banner motd #Unauthorized Access is Prohibited#

end

## Appendix B: Complete Distribution Switch DS1 Configuration

hostname DS1

ip routing

!

vlan 10

name Building_A_Users

vlan 20

name Building_B_Users

!

interface Vlan10

ip address 172.16.10.1 255.255.255.0

no shutdown

!

interface Vlan20

ip address 172.16.20.1 255.255.255.0

no shutdown

!

interface GigabitEthernet0/1

no switchport

ip address 10.0.1.2 255.255.255.252

no shutdown

!

spanning-tree mode pvst

spanning-tree vlan 10 priority 4096

spanning-tree vlan 20 priority 4096

!

router ospf 1

router-id 10.10.4.1

network 10.0.1.0 0.0.0.3 area 1

network 172.16.10.0 0.0.0.255 area 1

network 172.16.20.0 0.0.0.255 area 1

!

end

## Appendix C: IP Addressing Quick Reference Table

| Device | Interface | IP Address | Subnet Mask | Description |
| --- | --- | --- | --- | --- |
| R1 | Loopback0 | 10.10.1.1 | 255.255.255.255 | OSPF Router ID |
| R1 | GigabitEthernet0/0 | 10.0.1.1 | 255.255.255.252 | Link to DS1 |
| R1 | Serial0/0/0 | 192.168.1.1 | 255.255.255.252 | WAN to R2 |
| R2 | Loopback0 | 10.10.2.1 | 255.255.255.255 | OSPF Router ID |
| R2 | Serial0/0/0 | 192.168.1.2 | 255.255.255.252 | WAN to R1 |
| R2 | Serial0/0/1 | 192.168.1.5 | 255.255.255.252 | WAN to R3 |
| R3 | Loopback0 | 10.10.3.1 | 255.255.255.255 | OSPF Router ID |
| R3 | Serial0/0/1 | 192.168.1.6 | 255.255.255.252 | WAN to R2 |
| R3 | GigabitEthernet0/0 | 10.0.2.1 | 255.255.255.252 | Link to DS2 |
| DS1 | GigabitEthernet0/1 | 10.0.1.2 | 255.255.255.252 | Uplink to R1 |
| DS1 | VLAN 10 SVI | 172.16.10.1 | 255.255.255.0 | Gateway for VLAN10 |
| DS1 | VLAN 20 SVI | 172.16.20.1 | 255.255.255.0 | Gateway for VLAN20 |
| DS2 | GigabitEthernet0/1 | 10.0.2.2 | 255.255.255.252 | Uplink to R3 |
| DS2 | VLAN 30 SVI | 172.16.30.1 | 255.255.255.0 | Gateway for VLAN30 |
| DS2 | VLAN 40 SVI | 172.16.40.1 | 255.255.255.0 | Gateway for VLAN40 |