UNIVERSITY OF BUEA

PROJECT REPORT

Project Title

DESIGN AND CONFIGURATION OF A SECURED HIERARCHICAL NETWORK ARCHITECTURE with Cisco packet tracers.

STUDENT NAME:  LONGHO VITALIS NGONG

SUPERVISOR:    Dr. ACHANKENG PETER LEKE

CHAPTER ONE

INTRODUCTION

  - Background Study

Hierarchical network architecture is a structured design approach that organizes   network   devices and service to improve scalability, performance, and management. By segmenting a flat network into smaller, manageable blocks, this model restricts traffic, enhances security, and simplifies troubleshooting. In a Hierarchical model, the network is divided into distinct layers. Also   known as Tiers. These layers are connected with one another in the form of a hierarchy which enables the network to be divided into more manageable blocks and these blocks limit the local traffic to remain local even if it is broadcast traffic. A hierarchical model can be applied for both LAN and WAN network design.

Hierarchical network architecture, first put forth by Cisco in 2002, has now spread throughout the industry as a recommended practice for creating dependable, scale-able, and affordable networks. Early networks were flat and could only be stretched in one direction using hubs and switches, making it difficult to filter unwanted traffic and manage broadcasts. Response times would worsen as a network expanded in size. It was necessary to create a new network design, which led to the hierarchical strategy. Even while flat network designs are still used today, they are typically used for very small networks or designs that aim to reduce costs by utilizing a minimal number of routers or switches. A network is divided into layers by a hierarchical structure, and each layer has a set of functions that specify how it fits into the overall network. This gives a network designer the ability to select the best hardware, software, and features to fulfill a specific purpose for that network layer. Additionally, data management is far more effective. Local traffic in a hierarchical design only goes to a higher tier when it is destined for another network. [1]

Hierarchical Network Design is now considered to be the best practice industry-wide to design networks that are reliable, resilient, scale-able, and also cost effective. Initially, networks where deigned in a flat topology where the end devices were connected using Hubs and Switches. In order to add more devices or more users, more Switches/Hubs were added to the network. This Flat network design would cause a delay in the network if in case the network grows and also because of the use of Hubs and Switches it would be very difficult for the admins to control and limit the broadcast or filters the undesired traffic in network [2]

Figure 1.1: Flat Network Design

Due to the above limitations of the Flat Network Design, a Hierarchical Network Design Model was introduced. In a Hierarchical model, the network is divided into three distinct layers. These layers are connected with one another in the form of a hierarchy which enables the network to be divided into more manageable blocks and these blocks limit the local traffic to remain local even if it is broadcast traffic. A hierarchical Model can be applied for both LAN and WAN Network design.[3]

Figure 1.2: Hierarchical Network Design

A 3-tier hierarchical network topology consist of three layers.

1.1.1 Access Layer:

- The Access Layer commonly referred to as the network edge is where the end-user devices connect to the network.

- It provides high-bandwidth connectivity.

- It provides Layer 2 Switching capabilities.

- Services like Port Security, Quality of Service are used in this layer

Figure 1.3: Access layer design

1.1.2 Distribution Layer:

This layer basically provides policy-based connectivity and acts as a boundary between the Access Layer and the Core Layer. Data Filtering and Routing take place in this layer.

- The Distribution Layer is mainly responsible for collecting/aggregating data from the Switches of the Access Layer and distributing it to the rest of the network.

- It acts as a border as well as a connector to both the Access Layer and the Core Layer.

- The use of routing services like OSPF.

- It provides Redundancy and Load Balancing.

Figure 1.4: Distribution layer design

1.1.2 Core Layer:

This layer is often considered to be the Backbone of the network which provides fast transport between the switches present in the Distribution Layer of the network. The Core Layer is considered to be the Backbone of the network and acts as an aggregation point for multiple networks.

- It consists of high-speed network devices responsible for switching packets as fast as possible.

- It provides interconnectivity between the Distribution Layer devices.

- It provides reliability and fault tolerance to maximize performance.

Figure 1.5: Core layer design

This 3-Tier Hierarchical Network Design maximizes performance, scalability, and network availability and minimizes costs. However, Small Scale Enterprise Networks are not large enough, and therefore they use 2-Tier Hierarchical Network Design also known as. Collapsed Core Networks.

## 1.2 Collapsed Core Network Design:

A Collapsed Core is when the functions of the Distribution Layer and the Core Layer are implemented by a single device. This type of Network Design is used by Small Scale Enterprises to implement their networks as their network is not large enough, and they might be unable to bear the high cost of the network devices. Therefore, these Enterprises use the 2-Tier Hierarchical Network Design to reduce the network cost while maintaining most of the benefits, functions, and services offered by the 3-Tier Networks. [4]

CHAPTER TWO

LITERATURE REVIEW

2.1 Conceptual Framework

The hierarchical network design model, often referred to as the three-tier model, was popularized by Cisco Systems and has become the standard approach for designing enterprise networks. According to Tanenbaum and Wetherall (2011), hierarchical models simplify network design by breaking large, complex systems into smaller, manageable layers with well-defined roles and responsibilities. [6]

2.1.1 The three layers are:

- Core Layer: This is the backbone of the network. Its primary function is to provide fast, reliable transportation of data between distribution layer devices. The core layer is optimized for high-speed switching and routing. Redundancy is critical at this layer, as failure here affects all parts of the network. Devices at the core layer are typically high-performance routers or multilayer switches.

- Distribution Layer: This layer serves as an aggregation point for the access layer and enforces policies such as routing, filtering, and Quality of service. It connects the access layer to the core and handles inter VLAN routing, and traffic management. Multilayer switches or routers are commonly deployed here.

- Access Layer: This is the point at which end devices such as PCs, printers, and IP phones connect to the network. Access layer switches enforce security policies, provide VLAN segmentation, and support Power over Ethernet. Port security and STP are commonly configured at this layer.

The benefits of the hierarchical model include; Modularity (each layer can be upgraded independently), scalability (the network can grow without redesigning the whole architecture), and manageability (each layer has a distinct function, making troubleshooting more straightforward).

2.1.2 Open Shortest Path First (OSPF)

OSPF is a link-state routing protocol defined in RFC 2328 by the Internet Engineering Task Force (IETF). Unlike distance-vector protocols such as RIP, OSPF maintains a complete topological map of the network in a database called the Link-State Database (LSDB). Each router exchanges Link State Advertisements with its neighbors to populate this database, and then independently runs Dijkstra's SPF algorithm to compute the shortest path to each destination.

OSPF supports a two-level hierarchical structure of its own: the backbone area (Area 0) and non-backbone areas. All non-backbone areas must connect to Area 0, either directly or through virtual links. This design reduces LSA flooding and limits the size of routing tables within each area. Area Border Routers (ABR) connect multiple areas and summarize routing information between them. [7]

2.1.2.1 Key OSPF concepts include:

- Router ID: A unique 32-bit identifier for each OSPF router, usually the highest IP address on a loop-back interface.

- Hello Protocol: OSPF routers discover neighbors by exchanging Hello packets. Adjacencies are formed when parameters such as Hello/Dead intervals, Area ID, and authentication match.

- Designated Router (DR) and Backup Designated Router (BDR): On multi-access networks (e.g., Ethernet), OSPF elects a DR and BDR to reduce the number of LSA exchanges. All routers form adjacency only with the DR and BDR.

- OSPF Cost: The metric used by OSPF is called cost, calculated as 100 Mbps divided by the interface bandwidth. Lower cost paths are preferred.

- OSPF States: Routers transition through several states when forming adjacency: Down, Attempt, Init, 2-Way, ExStart, Exchange, Loading, and Full. [8]

2.1.3 Point-To-Point Protocol

PPP is a Layer 2 protocol standardized in RFC 1661. It is designed to encapsulate network layer packets over serial point-to-point links, providing a standardized method for two directly connected devices to communicate. PPP replaced the older SLIP (Serial Line Internet Protocol) due to its richer feature set, which includes:

- Multi-protocol support: PPP can carry multiple network layer protocols such as IP, IPX, and Apple Talk simultaneously.

- Error detection: PPP uses a 16-bit CRC for error checking.

- Authentication: PPP supports PAP (Password Authentication Protocol) and CHAP (Challenge Handshake Authentication Protocol). PAP sends credentials in plaintext, while CHAP uses MD5 hashing for a more secure exchange.

- Link Control Protocol (LCP): Used to establish, configure, and test the data link connection.

- Network Control Protocol (NCP): Used to establish and configure different network layer protocols.

CHAP authentication works through a three-step process:

- The authenticator sends a challenge message

- The peer responds with a hash value computed using the challenge and a shared secret, and

- The authenticator compares the response with its own computation. If they match, access is granted.[9]

2.1.4 Spanning Tree Protocol (SPT)

STP was developed by Radia Perlman in 1985 and later standardized as IEEE 802.1D. Its fundamental purpose is to prevent Layer 2 loops in networks with redundant paths. Without STP, a broadcast frame entering a loop would be forwarded indefinitely, saturating all network bandwidth and causing a broadcast storm.[11]

STP operates by electing a Root Bridge, which becomes the logical center of the spanning tree. All other switches compute a shortest path to the root bridge and block any redundant paths. The STP process involves:

- Root Bridge Election: The switch with the lowest Bridge Identifier (BID), composed of a priority value (default 32768) and the switch's MAC address, is elected as the root bridge.

- Root Port Selection: Each non-root switch selects a Root Port, which is the port with the lowest cost path to the root bridge.

- Designated Port Selection: On each network segment, the switch with the lowest path cost to the root bridge has its port designated as the Designated Port.

- Port States: STP ports transition through Blocking, Listening, Learning, and Forwarding states. Convergence to the Forwarding state can take up to 50 seconds in standard STP (IEEE 802.1D).

Rapid Spanning Tree Protocol (RSTP), defined in IEEE 802.1w, improves convergence time to less than one second by introducing new port roles and eliminating some of the listening and learning states.

## 2.2 Theoretical Framework

## 2.2.1OSI Reference MODEL

The Open Systems Interconnection (OSI) model, developed by the International Organization for Standardization (ISO), provides a conceptual framework for understanding how different network protocols interact. It consists of seven layers: Physical, Data Link, Network, Transport, Session, Presentation, and Application. The protocols discussed in this project operate at different OSI layers. Understanding the OSI model helps engineers troubleshoot by isolating problems to a specific layer, ensuring the correct protocol is examined.

| Protocol | OSI Layer | Function |
| --- | --- | --- |
| STP | Layer 2 (Data Link) | Loop prevention in switched networks |
| PPP | Layer 2 (Data Link) | WAN encapsulation and authentication |
| OSPF | Layer 3 (Network) | Dynamic routing and path selection |
| IP | Layer 3 (Network) | Logical addressing and packet forwarding |
| Ethernet | Layer 2 (Data Link) | Frame transmission on LAN links |

2.2.2 Network convergence Theory

Network convergence refers to the time it takes for all routers or switches in a network to agree on a consistent view of the topology after a change (such as a link failure). Fast convergence is critical for minimizing service disruption. OSPF achieves relatively fast convergence compared to distance-vector protocols because it floods link state changes throughout the area immediately upon detection. STP convergence in its original 802.1D form is slow (30-50 seconds), which motivated the development of RSTP and other improvements.

2.2.3 Redundancy and Fault Tolerance

Redundancy in network design refers to the use of multiple paths, devices, or links so that a single failure does not bring down the entire network. However, redundancy at Layer 2 (switches) without STP creates loops. Redundancy at Layer 3 (routers) is handled by OSPF ability to route around failed links. The interplay between STP and OSPF in a hierarchical design ensures both Layer 2 and Layer 3 redundancy operate correctly without interference.

2.3 Empirical Literature

2.3.1 Related studies

Oluwadare and Babatunde (2019) conducted a study on OSPF performance in a simulated enterprise network using Cisco Packet Tracer. Their findings showed that OSPF converged faster than RIP in networks with more than 10 routers and provided more accurate routing in the presence of multiple paths. They recommended OSPF for any network with more than two routers.

Musa et al. (2020) compared different WAN encapsulation protocols including PPP and HDLC in a simulated environment. Their results showed that PPP with CHAP authentication provided a significantly more secure connection than HDLC or PPP without authentication, with negligible performance overhead. They concluded that CHAP should be the default configuration for all PPP serial links in security-sensitive environments. [15]

Eze and Nwachukwu (2021) investigated the impact of STP on network performance in a topology with multiple switches and redundant paths. Their simulation demonstrated that enabling STP eliminated all broadcast loops within 45 seconds of network startup, while RSTP reduced this to under 2 seconds. The study recommended RSTP over legacy STP for modern enterprise deployments.

Abubakar et al. (2022) designed a hierarchical campus network for a Nigerian university using Cisco Packet Tracer, integrating OSPF and VLANs. Their study highlighted that the hierarchical model improved traffic management and allowed network administrators to isolate faults more efficiently than a flat design. They also noted that proper IP addressing and OSPF area planning were critical success factors.

These empirical works collectively support the design choices made in this project: using OSPF for dynamic routing, PPP with CHAP for secure WAN links, and STP for loop-free switching. They also validate the use of Cisco Packet Tracer as a reliable simulation tool for undergraduate network projects.

2.3.2 Gap in the literature

Despite the extensive literature on individual protocols, few studies have documented the simultaneous integration of OSPF, PPP, and STP within a single hierarchical network model with step-by-step configuration commands and verification outputs. Most existing works focus on one protocol at a time or use flat topology. This project fills that gap by providing an end-to-end, multi-protocol hierarchical network implementation, complete with configuration steps and verification evidence. [19]

CHAPTER THERR

METHODOLOGY AND DESIGN

3.1 Methodology

This project follows the Network Development Life Cycle (NDLC), a structured methodology for planning, designing, implementing, and operating network systems. The NDLC consists of the following phases:

- Analysis Phase: Requirements were gathered by identifying the network's functional goals, including the number of users, device types, routing requirements, WAN connectivity, and redundancy needs. The three-tier hierarchical model was selected as the architectural framework.

- Design Phase: Based on the requirements, a logical network topology was created. IP addressing schemes were planned for all interfaces, sub-nets were allocated for each layer, and protocol parameters were determined for OSPF, PPP, and STP.

- Simulation Phase: The physical and logical topology was constructed in Cisco Packet Tracer. All devices were placed, connected with appropriate cable types, and assigned IP addresses.

- Configuration Phase: Each device was configured via its CLI (Command Line Interface). OSPF was configured on all routers, PPP with CHAP was configured on serial links, and STP was configured on all switches.

- Verification Phase: The network was tested using Cisco IOS show commands and connectivity tests (ping and trace route) to verify that the protocols were functioning correctly.

- Documentation Phase: All configuration steps, commands, and outputs were documented with screenshots and explanations.

This iterative approach ensures that each phase builds on the previous one, and problems identified during verification are fed back into the design or configuration phases for correction.

## 3.2 Tools and Software

| TOOL/SOFTWARE | VERSION | PURPOSE |
| --- | --- | --- |
| Cisco packet tracer | 9.0.0.0910 | Primary networking simulation environment |
| Microsoft word/WPS | 2013 | Project documentation |
| Cisco IOS | 9.0.0.0810 | Router switches operating system |
| Windows 10 Pro OS | 10.0.0.19045 | Host operating system for simulation |

    - Justification For Cisco Packet Tracer

Cisco Packet Tracer was selected as the primary simulation tool for the following reasons:

- It is freely available to students through the Cisco Networking Academy.

- It supports all required Cisco IOS commands for OSPF, PPP, and STP configuration.

- It provides a visual, drag-and-drop interface for topology design.

- It allows real-time simulation and observation of packet flows.

- It is widely recognized in academic and professional circles as a valid network design tool.

## 3.3 Network Specifications

## 3.3.1 Functional requirement

| DEVICE | MODEL | QUANTITY | LAYER | ROLE |
| --- | --- | --- | --- | --- |
| router | Cisco 2911 | 3 | core | OSPF routing, PPP,WAN links |
| Multilayer Switch | Cisco 3560 | 2 | Distribution | Inter-VLAN routing, OSPF |
| Access Switch | Cisco2960 | 3 | access | STP,VLAN access |
| PC end device | Generic pc | 10 | access | End user Simulation |
| Server | Server PT | 2 | access | DNS/DHCP Simulation |
| Printer | Printer PT | 2 | Access | Documentation print out |
| Access point | Access point-PT | 1 | access | Wireless connection |
| Connection type | Copper straight-through | 16 | Access | Connect dissimilar device |
|  | Copper cross-over | 7 | Access | Connect similar device |
|  | Serial DCE | 2 | core | Relay or carrier of data |

3.4 DESIGN

Figure 3.1

3.4.1 The Three-Tier Structure

3.4.1.1 CORE LAYER TWO ROUTERS (R1&R2)

At the core layer the router (R1) and router (R2)   connect to each other through and OSPF, Point to Point link forming the backbone. They route to the internet .Each router connect to both multilayer switches through a primary link and a redundant cross link so that if one router or cable fails, traffic will still flow.

Serial Data Communication Equipment cable is used to connect two router each linking to Internet service provider and each line having the network IP address of 192.136.17./30 (192.136.17.19 & 192.136.17.19) and the other having the network 192.136.17.4/30. (192.136.17.4.6 & 192.136.17.4.5)

3.4.1.2 DISTRIBUTION LAYER (MLS1 & MLS2)

This is the heart of the design. The two multilayer switches links to each other by trucking and carrying all the VLANs which link to the router with the network IP address of 192.168.3.156/30(192.168.3.158 & 192.168.3.157) , 192.168.3.152/30 (192.168.3.153 & 192.168.3.154) and of 192.168.3.144/30(192.168.3.146 & 192.168.3.145) , 192.168.148/30 (192.168.149 & 192.168.150) respectively. The link up to the router using four link and link down to the access layer with six links all with copper cross-over cable. The multilayer switch one (MLS1) is the STP Root Bridge and active gateway. MLS2 is the standby for failover with no manual intervention.

3.4.1.3 ACCESS LAYER (IT DEPARTMENT, SERVER ROOM, FINANCE DEPARTMENT)

Each access switch is connect to active MLS1 and MLS2 with two link each but STP blocks the redundancy port to prevent loops if the primary link dies, the block port  activates with second.

3.5 REASON FOR THIS DESIGN

- No single point of failure where every devices has a redundant path.

- OSPF handle automatic re-routing if a link breaks

- HSRP ensure end devices never lose their default gateway

- STP prevents broadcasting storms from all the redundant loops.

In the layer, the following are the IP address and VLANs with their subnet mask are:

IP addresses for VLAN 10 are 192.168.1. 3/25, 192.168.1 4/25 192.168.1. 5/25, 192.168.1. 7/25 192.168.1. 8/25

IP addresses for VLAN 20 are 192.168.2. 2/29, 192.168.2. 3/29, 192.168.2. 4/29 192.168.2. 5/29.

IP addresses for VLAN 30 are 192.168.3. 2/29, 192.168.3. 3/25, 192.168.3. 4/25 192.168.3. 5/29.

3.4 Project Timeline (Gantt chart)

The following table represents the project timeline over a 127-days period:

Figure 3.2

CHAPTER FOUR

IMPLEMENTATION